MikroTik Dual WAN Policy Routing and Failover Configuration

Objective

Create a configuration where:

- PC1 uses ISP1 as the primary gateway

- PC2 uses ISP2 as the primary gateway

- If one ISP fails, traffic automatically switches to the backup ISP (failover)

Topology

IP Address Configuration

| ISP 1 | ISP2 | Mikrotik | PC1 | PC2 |
| --- | --- | --- | --- | --- |
| Ether 1 192.168.133.103/24 | Ether 1 192.168.133.103 | Ether 1 10.1.1.2/30 | Ether 1 192.168.1.254/24 | Ether 1 192.168.12.254/24 |
| Ether 2 10.1.1.1/30 | Ether 2 10.2.2.1/30 | Ether 2 10.2.2.2/30 |  |  |
|  |  | Ether 3 192.168.11.1/24 |  |  |
|  |  | Ether 4 192.168.12.1/24 |  |  |

Configure ISP 1 and ISP 2 : IP Address, DHCP Server, NAT

Configure Mikrotik IP Address, DHCP Server, NAT

Add Mangle rule to filter route every PC

/ip firewall mangle

add chain=prerouting src-address=192.168.10.0/24 action=mark-routing new-routing-mark=toISP1

add chain=prerouting src-address=192.168.20.0/24 action=mark-routing new-routing-mark=toISP2

Explanation

Mangle rules are used to mark packets based on source IP address.

- Network 192.168.10.0/24 receives the mark toISP1

- Network 192.168.20.0/24 receives the mark toISP2

The routing mark allows the router to select different gateways for different clients.

Configure Route and Failover

/ip route

PC1 Primary :

add dst-address=0.0.0.0/0 gateway=10.10.10.254 routing-mark=toISP1 distance=1 check-gateway=ping

PC1 Backup :

add dst-address=0.0.0.0/0 gateway=20.20.20.254 routing-mark=toISP1 distance=2

PC2 Primary : add dst-address=0.0.0.0/0 gateway=20.20.20.254 routing-mark=toISP2 distance=1 check-gateway=ping

PC2 Backup : add dst-address=0.0.0.0/0 gateway=10.10.10.254 routing-mark=toISP2 distance=2

Routing Explanation

Distance

Distance is used to setting route priority.

- distance=1 → primary route

- distance=2 → backup route

The router always chooses the route with the lowest distance value.

Check Gateway

check-gateway=ping

This feature checks whether the primary gateway is still reachable.

If the gateway does not respond :

- the route becomes inactive

- the router automatically switches to the backup route

Testing

We can see in the photo PC1 access internet from ISP1 and PC2 from ISP2, if the ISP1 trouble or have an problem PC1 will access internet from ISP2 so on.

Test if connection from ISP1 trouble :